UpstreamAI

Field Development Plan

Engineering & Financial Formula Reference

Version 1.0  •  March 2026

# 1. Overview

This document describes every calculation, formula, and algorithm embedded in the UpstreamAI FDP Decode application. The application covers nine ADNOC offshore fields and provides reservoir engineering, financial analysis, uncertainty quantification, and AI-confidence metrics across eleven interactive screens.

Architecture Overview

• Layer 1 – Executive Dashboard: portfolio-level NPV, capital efficiency, risk metrics

• Layer 2 – Asset Manager Dashboard: field-level KPIs, FDP Nerve Center

• Layer 3 – Reservoir Engineer Tools: decline curves, history matching, Monte Carlo

# 2. Asset Data Model

Each field in the AssetContext carries the following parameters that feed all formulas:

| ooip | MMbbl – Original Oil In Place |
| --- | --- |
| cumulativeOil | MMbbl – Cumulative oil produced to date |
| initialPressure | psi – Original reservoir pressure |
| pressure | psi – Current reservoir pressure |
| firstOilYear | Calendar year of first production |
| declineRate (Di) | Annual nominal decline rate (fraction/yr) |
| declineExponent (b) | Arps b-exponent (0 = exponential, 1 = harmonic) |
| gor | scf/bbl – Gas-Oil Ratio |
| opexPerBbl | $/bbl – All-in operating cost |
| remainingCapexMM | $MM – Remaining capital expenditure |
| fieldLifeYears | Remaining production life (years) |
| mobilityRatio | Water/oil mobility ratio (dimensionless) |
| dataCompleteness | Weighted data completeness score (0–100%) |
| historyMatchR2 | History match coefficient of determination R² (0–1) |

# 3. Reservoir Engineering Calculations

3.1  Recovery Factor (RF)

Measures what fraction of the Original Oil In Place has been recovered to date.

Formula:

RF (%) = (Np / N) × 100

| RF | Recovery Factor (%) |
| --- | --- |
| Np | Cumulative oil produced to date (MMbbl) |
| N | Original Oil In Place – OOIP (MMbbl) |

Application: Displayed on Dashboard, FDP Summary, and DeepDive Analytics screens

Implementation: calculateRecoveryFactor(cumulativeOil, ooip) in calculations.ts

3.2  Water Cut

Fraction of produced liquid that is water, expressed as a percentage.

WC (%) = Qw / (Qo + Qw) × 100

| WC | Water Cut (%) |
| --- | --- |
| Qo | Oil production rate (bbl/day) |
| Qw | Water production rate (bbl/day) |

Application: Dashboard KPI card and trend charts

Implementation: calculateWaterCut(waterBpd, oilBpd)

3.3  Arps Hyperbolic Decline Curve  (SPE-945)

Used to forecast future production rates. The hyperbolic form covers exponential (b = 0) through harmonic (b = 1) decline behaviour.

q(t) = qi / (1 + b × Di × t)^(1/b)

| q(t) | Production rate at time t (bbl/day) |
| --- | --- |
| qi | Initial production rate (bbl/day) |
| Di | Nominal annual decline rate (fraction/yr) |
| b | Arps exponent (0 ≤ b ≤ 1) |
| t | Time elapsed since start of decline (years) |

Special case  b = 0: q(t) = qi × exp(−Di × t)   [exponential decline]

Cumulative production under hyperbolic decline (b ≠ 0) for an annual forecast step is integrated analytically:

Np(t1→t2) = [qi^b / (Di × (1−b))] × [q(t1)^(1−b) − q(t2)^(1−b)]

Application: HistoryMatching chart, DeepDive P10/P50/P90 forecast, FDP production profile

Implementation: hyperbolicDecline(qi, Di, b, t) · generateAnnualForecast(...)

3.4  Reservoir Pressure – Material Balance Approximation

Simplified liquid-expansion material balance to estimate current average reservoir pressure from cumulative production.

P_current ≈ P_initial × (1 − Np / (N × ct × P_initial))

Or equivalently using a linear pressure depletion approximation:

P_current = P_initial × (1 − Np / N × depletion_factor)

| P_current | Estimated current reservoir pressure (psi) |
| --- | --- |
| P_initial | Original reservoir pressure (psi) |
| Np | Cumulative oil produced (MMbbl) |
| N | OOIP (MMbbl) |
| ct | Total compressibility (~1.5 × 10⁻⁵ psi⁻¹ default) |
| depletion_factor | Calibration scalar (default 0.5) |

Application: Dashboard current pressure KPI

Implementation: estimateReservoirPressure(initialPressure, ooip, cumulativeOil, oilFVF?, ct?)

3.5  Sweep Efficiency

Combines Craig's areal sweep efficiency with Dykstra-Parsons vertical heterogeneity to estimate displacement efficiency.

Areal Sweep (Craig correlation)

Ea = 0.54602789 + 0.03170817/M − 0.00509693×M + 0.11289012×ln(M)

| Ea | Areal sweep efficiency (fraction) |
| --- | --- |
| M | Mobility ratio = (krw/μw) / (kro/μo) |

Vertical Sweep (Dykstra-Parsons)

Ev = 1 − V_DP

| Ev | Vertical sweep efficiency (fraction) |
| --- | --- |
| V_DP | Dykstra-Parsons coefficient of variation (0–1, default 0.72) |

Overall Displacement Efficiency

E_total (%) = Ea × Ev × 100

Application: Dashboard Sweep Efficiency KPI

Implementation: calculateSweepEfficiency(mobilityRatio, dykstraParsonV?)

# 4. Financial Analysis Calculations

4.1  Net Present Value (NPV)

Full discounted cash flow using an annuity approximation when a detailed year-by-year forecast is not computed. Incorporates royalty, income tax, and remaining CAPEX.

Revenue and Cost Stream

Revenue/yr   = q × OilPrice × 365

Royalty/yr   = Revenue × royaltyRate         (default 5%)

OPEX/yr      = q × opexPerBbl × 365

EBITDA/yr    = Revenue − Royalty − OPEX

Tax/yr       = EBITDA × taxRate              (default 55%)

NetCF/yr     = EBITDA − Tax

Present Value Annuity (mid-year convention)

PV(annuity) = NetCF × [1 − (1+r)^(−n)] / r × (1+r)^0.5

Final NPV

NPV = PV(annuity) − CapEx_total

| q | Current oil production rate (bbl/day) |
| --- | --- |
| OilPrice | Flat oil price assumption ($/bbl, default $80) |
| royaltyRate | Government royalty fraction (default 0.05) |
| opexPerBbl | All-in operating cost ($/bbl) |
| taxRate | Effective income tax rate (default 0.55 for PSC) |
| r | Discount rate (default 10%) |
| n | Field life remaining (years) |
| CapEx_total | Remaining capital expenditure ($B) |

Output units: Billions USD ($B) for display; $MM internally

Application: Dashboard, FDP Summary, ExecutiveDashboard

Implementation: calculateNPVSimplified(...)  ·  calculateNPV(forecast, ...)

4.2  Internal Rate of Return (IRR)

Newton-Raphson root-finding on the NPV=0 equation. Cash-flow array starts with −CapEx in Year 0 followed by annual net cash flows.

NPV(r) = Σ [CF_t / (1+r)^t] = 0   →   solve for r

Newton-Raphson iteration

r_{n+1} = r_n − NPV(r_n) / NPV'(r_n)

NPV'(r) = −Σ [t × CF_t / (1+r)^(t+1)]

| CF_0 | −CapEx (initial investment, negative) |
| --- | --- |
| CF_t | Net cash flow in year t (positive) |
| r_0 | Initial guess (default 10%) |
| tol | Convergence tolerance (10⁻⁶) |
| maxIter | Maximum iterations (1000) |

Application: FDP Summary screen IRR metric

Implementation: calculateIRR(cashFlows, guess?)

4.3  Value Investment Ratio (VIR) / Capital Efficiency

Measures NPV generated per dollar of capital invested.

VIR = NPV / PV(CapEx)

PV(CapEx) discounts the remaining capex to present value:

PV(CapEx) = CapEx_MM / (1 + r)^(n/2)

| NPV | Net Present Value ($B) |
| --- | --- |
| PV(CapEx) | Present value of remaining capital expenditure ($B) |
| r | Discount rate (10%) |
| n | Field life remaining (years); n/2 uses midpoint approximation |

Interpretation: > 1.0x means investment creates more than $1 of NPV per $1 invested

Application: Dashboard KPI, ExecutiveDashboard portfolio view

Implementation: calculateCapitalEfficiency(npvB, pvCapexB)

4.4  Risk-Adjusted Return (RAR)

Portfolio-level ratio of P50 NPV to total capital deployed.

RAR = P50_NPV_MM / TotalCapEx_MM

Application: Executive Dashboard portfolio risk metrics

Implementation: calculateRiskAdjustedReturn(p50NpvMM, totalCapexMM)

# 5. Uncertainty Quantification

5.1  Monte Carlo NPV Simulation

A stochastic NPV model sampling key reservoir and economic uncertainties. Runs n = 5,000 iterations per analysis.

Sampling Distributions

| OOIP | Triangular(low, mode, high) — reflects structural uncertainty |
| --- | --- |
| Recovery Factor | Triangular(low, mode, high) — EOR upside / base / downside |
| Oil Price | Log-normal(μ, σ) where σ = mean × CV (CV default 20%) |
| OPEX | Normal(mean, std) clamped at 0 — cost uncertainty |
| CapEx | Normal(mean, std) clamped at 0 — development cost overrun |

Triangular Distribution Inverse CDF

u < Fc: x = low + √( u × (high−low) × (mode−low) )

u ≥ Fc: x = high − √( (1−u) × (high−low) × (high−mode) )

Fc = (mode − low) / (high − low)

Log-Normal Sampling (Box-Muller)

z = √(−2 ln U1) × cos(2π U2)    [standard normal]

σ_ln = √( ln(1 + CV²) )

μ_ln = ln(mean) − σ_ln²/2

x = exp(μ_ln + σ_ln × z)

Per-Iteration NPV

Np_iter = OOIP_iter × RF_iter     [MMbbl ultimate recovery]

Revenue  = Np_iter × 1e6 × Price_iter / fieldLife

NetCF    = (Revenue − OPEX_iter × Np_iter/fieldLife × 1e6) × (1−tax) − CapEx_iter

NPV_iter = NetCF × annuity_factor(r, fieldLife)

Output Statistics

| P90 (optimistic) | 10th percentile of sorted NPV distribution |
| --- | --- |
| P50 (base case) | 50th percentile of sorted NPV distribution |
| P10 (pessimistic) | 90th percentile of sorted NPV distribution |

Application: DeepDive Analytics – Monte Carlo histogram and P10/P50/P90 cards

Implementation: runMonteCarloNPV(inputs, n)  ·  getPercentile(sorted, p)  ·  buildHistogramBins(values, bins)

5.2  One-At-a-Time (OAT) Sensitivity – Tornado Chart

Perturbs each input parameter ±20% (holding all others fixed) and measures the resulting NPV swing. Sorted by absolute impact for tornado chart display.

Swing_i = NPV(+20%_i) − NPV(−20%_i)

Sensitivity_i = Swing_i / BaseNPV × 100  [%]

| Oil Price | ±20% of base price ($80/bbl) |
| --- | --- |
| Permeability | ±15% proxy affecting production rate |
| OOIP | ±15% structural uncertainty |
| Recovery Factor | ±15% EOR efficiency uncertainty |
| OPEX | ±20% cost uncertainty |
| Discount Rate | ±20% of base 10% |
| Gas Revenue | ±20% of gas contribution |
| Decline Rate (Di) | ±20% of field Di |

Application: Uncertainty screen – Tornado Chart

Implementation: calculateOATSensitivity(baseNPV_MM, parameters)

# 6. History Matching Quality Metrics

6.1  Coefficient of Determination (R²)

R² = 1 − SS_res / SS_tot

SS_res = Σ (y_obs_i − y_sim_i)²

SS_tot = Σ (y_obs_i − ȳ_obs)²

| R² | Goodness of fit (1.0 = perfect match) |
| --- | --- |
| y_obs_i | Observed production/pressure value at step i |
| y_sim_i | Simulated model value at step i |
| ȳ_obs | Mean of observed values |

6.2  Root Mean Square Error (RMSE)

RMSE = √( Σ (y_obs_i − y_sim_i)² / n )

Normalisation: Sometimes expressed as a fraction of the mean observed value

6.3  Mean Absolute Percentage Error (MAPE)

MAPE (%) = (100/n) × Σ |y_obs_i − y_sim_i| / |y_obs_i|

6.4  Composite History Match Quality Score

Combines production match and pressure match quality into a single 0–100% score.

HMQ = 0.6 × R²_prod × 100  +  0.4 × R²_press × 100   (if pressure data available)

HMQ = R²_prod × 100                                    (production only)

Application: History Matching screen – Quality Score card

Implementation: calculateHistoryMatchQuality(observedProd, simulatedProd, observedPressure?, simulatedPressure?)

6.5  Historical Chart Data Generation

Synthetic historical data is generated using Arps decline with seasonal perturbations (sinusoidal, no random noise) so the chart remains deterministic and reproducible across re-renders.

q_seasonal(t) = q_decline(t) × (1 + 0.04 × sin(2π × month/12))

q_simulated(t) = q_decline(t)   [clean model curve]

Application: History Matching screen – production match chart

Implementation: generateMatchChartData(currentRate, Di, b, firstOilYear, ...)

# 7. Oil Price Scenario Modelling

7.1  Ornstein-Uhlenbeck Mean-Reversion

Deterministic price path (no random term) using mean-reversion dynamics. Models the tendency of oil prices to revert toward a long-run equilibrium.

P(t) = P_eq + (P_0 − P_eq) × exp(−κ × t)

| P(t) | Price at year t ($/bbl) |
| --- | --- |
| P_0 | Current oil price ($/bbl, default $78) |
| P_eq | Long-run equilibrium price ($/bbl, default $72) |
| κ | Mean-reversion speed (yr⁻¹, default 0.20) |
| t | Years from today |

Scenario Bands

P_bull(t)  = P(t) × (1 + uplift)   [upside scenario, default +25%]

P_base(t)  = P(t)                   [base / central scenario]

P_bear(t)  = P(t) × (1 − haircut)  [downside scenario, default −30%]

Application: Market Volatility screen – price scenario fan chart

Implementation: generatePriceScenarios(current, equilibrium, κ, startYear, years, uplift, haircut)

# 8. Data Health & Completeness

8.1  Weighted Data Completeness

Each data category is assessed for the number of validated items versus total required items. Category scores are averaged weighted by importance.

Completeness_i (%) = validatedItems_i / totalItems_i × 100

OverallCompleteness = Σ (Completeness_i × weight_i) / Σ weight_i

| Static Model | weight = 0.25 (structural framework – highest criticality) |
| --- | --- |
| Well Data | weight = 0.20 |
| Petrophysical Logs | weight = 0.20 |
| Geological Interpretations | weight = 0.15 |
| Geophysical Data | weight = 0.10 |
| Production History | weight = 0.10 |

Application: Data Health screen – Overall Completeness gauge

Implementation: calculateWeightedCompleteness(categories)

8.2  Critical Gaps Count

CriticalGaps = Σ criticalMissing_i  (across all categories)

8.3  Validation Aggregation

TotalValidated = Σ validatedItems_i

TotalRequired  = Σ totalItems_i

Application: Data Health screen – validated/total counter

# 9. FDP Completion Score

The overall FDP completion percentage is calculated as a weighted average of key development milestones, where each milestone is assigned a relative importance weight.

FDPCompletion (%) = Σ (progress_i × weight_i) / Σ weight_i

| Milestone | Weight |
| --- | --- |
| Data Acquisition & QC | 0.15 |
| Static Model Build | 0.20 |
| Dynamic Simulation | 0.25 |
| Uncertainty Quantification | 0.15 |
| Development Concept Selection | 0.10 |
| Economics & Approvals | 0.15 |

Each milestone progress value (0–100%) is tracked against its status: Not Started = 0%, In Progress = 0–99%, Completed = 100%.

Application: Executive Dashboard – FDP Completion KPI

Implementation: calculateFDPCompletion(milestones)

# 10. AI Confidence Score

A composite metric measuring the reliability of AI-generated insights based on three orthogonal quality dimensions.

DataQuality     = dataCompleteness (%)

ModelConfidence = historyMatchR2 × 100

UncertCoverage  = uncertaintyCoverage × 100     (default 75%)

AIConfidence_overall = 0.40 × DataQuality + 0.35 × ModelConfidence + 0.25 × UncertCoverage

| DataQuality | Weighted data completeness score (0–100%) |
| --- | --- |
| ModelConfidence | History match R² scaled to percentage |
| UncertCoverage | Fraction of uncertainty parameter space modelled (0–1) |

Sub-scores (displayed on Insights screen)

InsightReliability = 0.5 × ModelConfidence + 0.3 × UncertCoverage + 0.2 × DataQuality

Application: Insights screen – AI confidence gauge; FDP Summary – model confidence

Implementation: calculateAIConfidence(dataCompleteness, historyMatchR2, uncertaintyCoverage)

# 11. Portfolio Analytics

11.1  Portfolio NPV Aggregation

NPV_portfolio = Σ NPV_field_i     [sum across all active fields]

Application: Executive Dashboard – Total Portfolio Value

Implementation: aggregatePortfolioNPV(fieldNPVs)

11.2  Portfolio Capital Efficiency

VIR_portfolio = NPV_portfolio / Σ PV(CapEx_field_i)

Application: Executive Dashboard – Portfolio VIR

Implementation: portfolioCapitalEfficiency(fields)

# 12. ADNOC Field Reference Data

The following table summarises the key engineering parameters for each of the nine ADNOC offshore fields modelled in the application.

| Field | OOIP (MMbbl) | RF% | Di (/yr) | b | GOR (scf/bbl) | OPEX ($/bbl) | Field Life (yr) | HM R² |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Zakum | 21,000 | 47% | 0.035 | 0.45 | 380 | $4.20 | 25 | 0.94 |
| Lower Zakum | 8,500 | 38% | 0.040 | 0.40 | 350 | $4.80 | 22 | 0.92 |
| Umm Shaif | 6,200 | 44% | 0.038 | 0.42 | 420 | $5.10 | 18 | 0.95 |
| Satah | 2,800 | 38% | 0.045 | 0.38 | 300 | $6.20 | 15 | 0.91 |
| Nasr | 1,900 | 28% | 0.052 | 0.35 | 260 | $7.50 | 20 | 0.88 |
| Umm Al Dalkh | 1,600 | 26% | 0.055 | 0.32 | 240 | $8.20 | 14 | 0.87 |
| Sarb | 1,200 | 26% | 0.058 | 0.30 | 275 | $8.80 | 12 | 0.86 |
| Umm Lulu | 1,500 | 18% | 0.030 | 0.48 | 310 | $6.80 | 28 | 0.96 |
| Abu Al Bukhoosh | 900 | 39% | 0.065 | 0.25 | 220 | $9.50 | 10 | 0.83 |

# 13. Display Formatting

| formatNPV(npvB) | "$X.XX B" or "$XXX M" — auto-scales billions/millions |
| --- | --- |
| formatProductionRate(bpd) | "XXX,XXX bbl/day" with locale comma formatting |
| formatReserves(mmbbl) | "X,XXX MMbbl" with comma formatting |

# 14. References

• Arps, J.J. (1945). Analysis of Decline Curves. SPE-945-PA, Trans. AIME 160, 228–247.

• Craig, F.F. (1971). The Reservoir Engineering Aspects of Waterflooding. SPE Monograph Vol. 3.

• Dykstra, H. & Parsons, R.L. (1950). The Prediction of Oil Recovery by Water Flood. Secondary Recovery of Oil in the United States, API.

• Ahmed, T. (2010). Reservoir Engineering Handbook, 4th Ed. Gulf Professional Publishing.

• Ornstein, L.S. & Uhlenbeck, G.E. (1930). On the Theory of the Brownian Motion. Physical Review 36, 823.

• Trigeorgis, L. (1996). Real Options: Managerial Flexibility and Strategy in Resource Allocation. MIT Press.

Generated by UpstreamAI FDP Decode  •  All formulas implemented in src/app/utils/calculations.ts